### Serpentine Optical Interferometer

DIY Rough Waveguide & Diffraction Experiment

#### Project Overview

A low-budget optical experiment using a 3D-printed Serpentine channel designed as a simple optical interferometer/waveguide. Laser light passes through the serpentine path and interacts with diffraction grating films at the input and/or output to observe interference and diffraction patterns.

Reference Design: Serpentine Optical Interferometer – Reference Drawings (rev.1)

#### Experimental Setup

- Laser: 633 nm Red Laser Pointer (incident distance ≈ 50 cm)

- Serpentine Channel: Black 3D-printed PLA

  - Size: 150 × 100 × 5 mm

  - Channel: Width 2.0 mm × Depth 1.5 mm

  - Bend radius: 20 mm

  - Mask angle: ±23.5°

- Diffraction Film: Educational grating film from Hanjin Science (estimated 500 lines/mm)

- Observation: White paper (A4) to wall (2.0–4.0 m distance)

#### Key Findings & Optimal Configuration

Best Configuration (Maximum Satellites)

- Output: Two diffraction grating films, both tilted in diamond (rhombus) orientation

- Input: Parallel or slightly tilted

- Result: Clear central spot + 5+ satellite spots scattered randomly near the main beam

Observed Phenomena

- Single film: Central spot (0th order) + linear diffraction (1st order)

- Film tilt (≈23.5°–35°): Dramatically improves 0th order efficiency

- Input & Output films in same orientation: Light survives / 90° crossed: Severe light loss

- After passing through Serpentine: Multiple modes and multiple internal paths create satellite spots (evidence of multi-path interference)

#### Photos (Recommended to upload)

- Overall experimental setup

- Output with two diamond-tilted films (optimal condition)

- Wall projection at 2.0–2.5 m (central spot + satellites) — preferably long-exposure shots

- Comparison of different film combinations (satellite count variation)

- Serpentine channel close-up

- Reference drawings (PDF)

(Use the clearest long-exposure photos showing multiple satellites, especially images 354.jpg, 355.jpg, 356.jpg, etc.)

#### Conclusions & Limitations

What We Confirmed

- The serpentine channel successfully creates multiple optical paths, leading to interference effects (satellite spots).

- Alignment, film tilt, and grating orientation are extremely sensitive — typical of DIY optics.

- Rough but meaningful evidence that the structure behaves as a very crude optical interferometer/waveguide.

Limitations

- Channel width (2.0 mm) is too wide for clean single-mode propagation → multi-mode behavior dominates.

- Low-cost educational grating films have limited efficiency.

- Very sensitive alignment makes reproducible results difficult without proper mounts.

Result Level: A very rough but directionally successful DIY experiment that matches our initial rough predictions.

#### Future Improvements

- Add convex lens at input for better beam focusing into the channel

- Build alignment jigs / fine adjustment stages

- Add collimation lens or aperture at output

- Introduce polarizing films

- Improve internal surface quality (reflective coating or smoother printing)

#### Project Files

- STL/ — 3D print files

- Drawings/ — Reference PDFs

- Photos/ — Experimental results

- README.md

Short Project Description (for GitHub repo description)

Low-budget experiment exploring multi-path interference using a 3D-printed serpentine optical waveguide and diffraction grating films. Observed multiple satellite spots indicating crude interference effects.